1. The library in your .blend is stale — my top suspect for "the fix didn't work."

Refresh From Folder imports files it doesn't already have. An asset already in the scene keeps the mesh it was first imported with — so a mesh built by the old UV-dropping code stays broken no matter how many times you refresh. Check: select a module → Object Data Properties (green triangle) → UV Maps. Empty means you're still looking at the old mesh. Fix: delete that library collection in the Outliner, then Refresh From Folder.

2. The material reads Generated or Object coordinates, not UV.

This one hides perfectly: the UV data stays correct the whole time, but the texture is positioned from the bounding box, which the add-on changes when it scales the asset into the 3 m slot. Direct import doesn't scale, so it looks right. Check: Shading tab — is the Image Texture's Vector input wired from a Texture Coordinate node's Generated/Object socket? Fix: use the UV socket, or a UV Map node.

3. Slot fitting is rescaling your asset.

Width fit scales uniformly so the asset becomes 3 m wide. UVs are untouched, but a tiled texture is then that much bigger or smaller per metre than in the file. An asset authored at exactly 3 m imports at scale 1 and nothing moves; a 4 m asset shrinks to 0.75×. Check: the import message says scaled x0.375 etc. Fix: author at 3 m, or set Fit to None.

4. Fit is set to Stretch.

Stretch scales X and Z by different amounts — the geometry is distorted, so the texture is too. Use Width.

5. Object scale isn't 1.

Texture stretches with the object; non-uniform scale stretches it unevenly. Fix: Ctrl+A → Scale.

6. Image extension is Extend, not Repeat.

Only bites once UVs go outside 0–1 — which the new Spread button does deliberately. Extend smears the edge pixel instead of tiling. Fix: Image Texture node → Extension → Repeat.

7. Duplicate materials (BrickMat.001, .002).

Every import makes another copy, so fixing the mapping on one doesn't reach the others, and some modules look right while others don't.

8. Wrong UV layer.

If the material names a specific layer and that name didn't survive the import, it silently falls back to a different one. Check: more than one entry under UV Maps.

9. The texture image was never copied.

Add Asset File copies your .obj and .mtl but not the image they reference — still an open gap I offered to close. This shows as pink or blank rather than wrong scale, so it's probably not your symptom.